RFID  READER

13.56MHz Reader/Writer

MIRT-1

User Manual

Version 2.3

CONTENT

1. 	GENERAL INFORMATION	5

2.	TYPES AND EXPLANATION	6

3.	CONNECTING TO PC	6

3.1 	MIRT-1-RS232	6

3.2 	MIRT-1-USB	7

4.	SDK	8

5.	DEMO	8

5.1 	Online	8

5.2 	SHC1102	9

5.3 	UltraLight	9

5.4 	Mifare_1K	10

5.5 	Mifare_4K	10

5.6 	Mifare_ProX	11

5.7 	TRH1064	11

5.9 	SRIX4K	12

5.10 	AT88RF020	13

5.11 	ISO14443B-4 Protocol Smart Card	13

5.12 	I.CODE SLI	14

5.13 	Tag_IT	14

5.14 	SRF55V02P	15

5.15 	SRF55V10P	15

5.16 	Pass_Through	16

6. 	DLL INFORMATION	17

6.1 	SYSTEM FUNCTION	17

6.1.1 	INT WINAPI LIB_VER	17

6.1.2 	INT WINAPI RF_INIT_COM	17

6.1.3 	INT WINAPI RF_CLOSEPORT	17

6.1.4 	INT WINAPI RF_GET_MODEL	17

6.1.5 	INT WINAPI RF_INIT_DEVICE_NUMBER	17

6.1.6 	INT WINAPI RF_GET_DEVICE_NUMBER	18

6.1.7 	INT WINAPI RF_INIT_TYPE	18

6.1.8 	INT WINAPI RF_ANTENNA_STA	18

6.1.9 	INT WINAPI RF_LIGHT	18

6.1.10 	INT WINAPI RF_BEEP	18

6.2 	DES FUNCTION	19

6.2.1 	INT WINAPI DES_ENCRYPT	19

6.2.2 	INT WINAPI DES_DECRYPT	19

6.3 	ISO14443A FUNCTION	19

6.3.1 	UltraLight	19

6.3.1.1 	INT WINAPI RF_REQUEST	19

6.3.1.2 	INT WINAPI INT RF_UL_SELECT	20

6.3.1.3 	INT WINAPI RF_M1_READ	20

6.3.1.4 	INT WINAPI INT RF_UL_WRITE	20

6.3.1.5 	INT WINAPI RF_HALT	20

6.3.2 	Mifare_Std	21

6.3.2.1 	INT WINAPI RF_REQUEST	21

6.3.2.2 	INT WINAPI RF_ANTICOLL	21

6.3.2.3 	INT WINAPI RF_SELECT	21

6.3.2.4 	INT WINAPI RF_M1_AUTHENTICATION2	22

6.3.2.5 	INT WINAPI RF_M1_READ	22

6.3.2.6 	INT WINAPI RF_M1_WRITE	22

6.3.2.7 	INT WINAPI RF_M1_INITVAL	22

6.3.2.8 	INT WINAPI RF_M1_READVAL	23

6.3.2.9 	INT WINAPI RF_M1_INCREMENT	23

6.3.2.10 INT WINAPI RF_M1_DECREMENT	23

6.3.2.11 INT WINAPI RF_M1_RESTORE	23

6.3.2.12 INT WINAPI RF_M1_TRANSFER	23

6.3.2.13 INT WINAPI RF_HALT	24

6.3.3 	Mifare_DESFire	24

6.3.3.1 	INT WINAPI RF_DESFIRE_RST	24

6.3.3.2 	INT WINAPI RF_COS_COMMAND	24

6.3.4 	Mifare_ProX	25

6.3.4.1 	INT WINAPI RF_TYPE_RST	25

6.3.4.2 	INT WINAPI RF_ COS_COMMAND	25

6.3.4.3 	INT WINAPI RF_CL_DESELECT	25

6.3.5 	SHC1102	25

6.3.5.1 	INT WINAPI RF_REQUEST	25

6.3.5.2 	INT WINAPI RF_ SHC1102_AUTH	26

6.3.5.3 	INT WINAPI RF_ SHC1102_READ	26

6.3.5.4 	INT WINAPI RF_ SHC1102_WRITE	26

6.4 	ISO14443B FUNCTION	27

6.4.1 	THR1064	27

6.4.1.1 	INT WINAPI RF_TYPEB_RST	27

6.4.1.2 	INT WINAPI RF_ THR1064_READ	27

6.4.1.3 	INT WINAPI RF_ THR1064_WRITE	27

6.4.1.4 	INT WINAPI RF_ THR1064_CHECK	27

6.4.2 	AT88RF020	28

6.4.2.1 	INT WINAPI RF_ TYPEB_RST	28

6.4.2.2 	INT WINAPI RF_ AT020_CHECK	28

6.4.2.3 	INT WINAPI RF_ AT020_COUNT	28

6.4.2.4 	INT WINAPI RF_ AT020_READ	28

6.4.2.5 	INT WINAPI RF_ AT020_WRITE	29

6.4.2.6 	INT WINAPI RF_ AT020_LOCK	29

6.4.2.7 	INT WINAPI RF_ AT020_DESELECT	29

6.4.3 	SR176SRIX4K	29

6.4.3.1 	INT WINAPI RF_ST_SELECT	29

6.4.3.2 	INT WINAPI INT_RF_SR176_READBLOCK	29

6.4.3.3 	INT WINAPI INT_RF_SR176_WRITEBLOCK	30

6.4.3.4 	INT WINAPI INT_RF_SR176_PROTECTBLOCK	30

6.4.3.5 	INT WINAPI INT_RF_SRIX4K_ GETUID	30

6.4.3.6 	INT WINAPI INT_RF_SRIX4K_ READBLOCK	30

6.4.3.7 	INT WINAPI INT_RF_SRIX4K_ WRITEBLOCK	31

6.4.3.8	INT WINAPI INT_RF_SRIX4K_ PROTECTBLOCK	31

6.4.3.9 	INT WINAPI RF_ST_COMPLETION	31

6.4.4 	TYPE_B SmartCard	32

6.4.4.1 	INT WINAPI RF_ TYPEB_RST	32

6.4.4.2 	INT WINAPI RF_ COS_COMMAND	32

6.4.4.3 	INT WINAPI RF_ CL_DESELECT	32

6.5 	ISO15693 FUNCTION	33

6.5.1	INT WINAPI ISO15693_INVENTORY	33

6.5.2 	INT WINAPI ISO15693_INVENTORYS	33

6.5.3 	INT WINAPI ISO15693_GET_SYSTEM_INFORMATION	33

6.5.4 	INT WINAPI ISO15693_SELECT	34

6.5.5 	INT WINAPI ISO15693_RESET_TO_READY	34

6.5.6 	INT WINAPI ISO15693_STAY_QUIET	34

6.5.7 	INT WINAPI ISO15693_GET_BLOCK_SECURITY	34

6.5.8 	INT WINAPI ISO15693_READ	35

6.5.9 	INT WINAPI ISO15693_WRITE	35

6.5.10 	INT WINAPI ISO15693_LOCK_BLOCK	36

6.5.11 	INT WINAPI ISO15693_WRITE_AFI	36

6.5.12 	INT WINAPI ISO15693_LOCK_AFI	36

6.5.13 	INT WINAPI ISO15693_WRITE_DSFID	37

6.5.14 	INT WINAPI ISO15693_LOCK_DSFID	37

6.6 	Function of Infineon Electric Tag	38

6.6.1 	INT WINAPI SRF55VP_READ	38

6.6.2 	INT WINAPI SRF55VP_WRITEBYTE	38

6.6.3 	INT WINAPI SRF55VP_WRITE	38

6.6.4 	INT WINAPI SRF55VP_WRITE_REREAD	39

6.7 	PASS THROUGH FUNCTION	39

6.7.1 	INT WIN API RF_TRANSCEIVE1	39

- 1. GENERAL INFORMATION

- RS232 or USB Interface

- DC4.5V ~ DC5.5V VDD Operating

- Windows 32 Operating Systems Compatibility

- 13.56MHz RF Operating Frequency

- ISO14443A ISO1443B ISO15693 Protocols

- 150MA Working Current

- Operating Temperature Range:  -20°C ~ +50°C

- Storage Temperature Range:	-25°C ~ +60°C

- Dimension: 110 × 81 × 26 mm

- Weight: 100g

- TYPES AND EXPLANATION

MIRT-1 series readers are in accord with ISO14443A, ISO14443B and ISO15693

protocols, and are classified as follows

|  | MIRT-1 | MIRT-1A | MIRT-15693 | MIRT-1E |  |
| --- | --- | --- | --- | --- | --- |
| ISO14443A | √ | √ |  | √ |  |
| ISO14443B |  |  |  | √ |  |
| ISO15693 |  |  | √ |  |  |

- CONNECTING TO PC

- 3.1 MIRT-1-RS232

The PS/2 port power to Reader

- 3.2 MIRT-1-USB

MIRT-1-USB Reader is USB bridge to COM way. Connect MIRT-1 to the USB port of PC, after installing the driver will come out a virtual COM, the operations hereafter are as same as MIRT-1-RS232.

You can find the virtual COM number on the “Device Manager ” as follows:

- SDK

Responding InstDemo.exe to install the DEMO software and the DLL of the reader to PC, and create corresponding logo on the desk.

The default installation directory is C:\RFREADER, including the following content:

C:\RFREADER\Examples       	Sample source code

C:\RFREADER\UsbDriver  	USB interface driver

C:\RFREADER\ICTransfer.exe 	DEMO software

C:\RFREADER\MasterRD.dll	Reader interface library with application

C:\RFREADER\MasterCOM.dll 	Connect and transfer data with COM device.

C:\RFREADER\SL_Paper.dll    	DEMO software background library

C:\RFREADER\RFHELP.chm  	DLL explanations at chm format

C:\RFREADER\AppConfig.ini   	DEMO software configuration files

- DEMO

This software run on Win32 system, and need 1024 x 768 dpi at least

- 5.1 Online

Choose the correct COM number, click [Connect] button to connect the Reader to

PC. Click [Read] the product information button, you can check the specific type of

the Reader and the supported cards.

- 5.2 SHC1102

Click [Request] button to obtain the card serial number.

Input the correct key to read/write the card

.

- 5.3 UltraLight

Click the [Request] button to obtain the card Serial Number.

Choose the corresponding address to read/write the card.

- 5.4 Mifare_1K (STD S50)

Click the [Request] button to obtain the card serial number.

Input the correct password to read, write, increase or decrease the card.

- 5.5 Mifare_4K (STD S70)

Click the [Request] button to obtain the card serial number.

Input the correct password to read, write, increase or decrease the card.

- 5.6 Mifare_ProX

Click [Reset] button to obtain the serial number and the reset information of the

card according to ISO14443-4 protocol.

Input the COS command, click [Send] button to commute data to card.

- 5.7 TRH1064

Click [Request] button to obtain the card serial number.

Hereafter can read, write and validate.

5.8 SR176

Click [Req] button to obtain the ID number of the card.

Then you can read, write and lock blocks of the card.

- 5.9 SRIX4K

Click [Req] button to obtain the ID number of the card and click [Read UID]

to obtain the UID of the card.

Then you can read, write and lock blocks of the card.

- 5.10 AT88RF020

Click [ReqB] button to obtain the serial number of the card.

After check password, you can read, write, signature and lock blocks of the

card.

- 5.11 ISO14443B-4 Protocol Smart Card

Click [Reset] button to obtain the serial number and the reset information of the

card according to ISO14443-4 protocol.

Input the COS command, click [Send] button to commute data to card.

- 5.12 I.CODE SLI

Click [INVENTORY] button to obtain the serial number of the card. You can

operate 4 cards at most.

Choose certain card according to the UID to read or write.

- 5.13 Tag_IT

Click [INVENTORY] button to obtain the serial number of the card. You can

operate 4 cards at most.

Choose certain card according to the UID to read/write.

- 5.14 SRF55V02P

Click [INVENTORY] button to obtain the serial number of the card. You can

operate 4 cards at most.

Choose certain card according to the UID to read/write.

- 5.15 SRF55V10P

Click [INVENTORY] button to obtain the serial number of the card. You can

operate 4 cards at most.

Choose certain card according to the UID to read/write.

- 5.16 Pass_Through

In this windows, input parameters according to ISO14443B and ISO15693 			protocol, click [Transceive] button to get response data from tag

CRC bytes is auto managed by reader, it will not be contained in the stream

- 6. DLL INFORMATION

All types of readers have System function and encrypt function.

Whether readers support other functions depends on their specific types.

- 6.1 SYSTEM FUNCTION

- 6.1.1 INT WINAPI LIB_VER

Function:  	Get DLL Version

Prototype: 	int WINAPI lib_ver (unsigned int *pVer)

Parameter: 	pVer: 	[OUT] 	DLL version

Return:   	return 0 if successful

- 6.1.2 INT WINAPI RF_INIT_COM

Function: 	Connect

Prototype: 	int WINAPI rf_init_com (int port, long baud)

Parameter: 	port: 	[IN] 	serial port number

baud: 	[IN] 	communication baud rate, 4800 ~ 115200 bps

Return:   	return 0 if successful

- 6.1.3 INT WINAPI RF_CLOSEPORT

Function: 	Disconnect

Prototyp: 	int WINAPI rf_ClosePort(void)

Return:   	return 0 if successful

- 6.1.4 INT WINAPI RF_GET_MODEL

Function: 	Get Device Type

Prototype: 	int WINAPI rf_get_model (unsigned short  icdev,

unsigned char  *pVersion,

unsigned char  *pLen)

Parameter: 	icdev: 	[IN]   	Device ID

pVersion: 	[OUT] 	response information

pLen:    	[OUT] 	length of response information

Return:   	return 0 if successful

- 6.1.5 INT WINAPI RF_INIT_DEVICE_NUMBER

Function: 	Designate Device ID

Prototype: 	int WINAPI rf_init_device_number (unsigned short icdev)

Parameter:	icdev:  	[IN] 	Device ID

Return:   	return 0 if successful

- 6.1.6 INT WINAPI RF_GET_DEVICE_NUMBER

Function:  	Read Device ID

Prototype: 	int WINAPI rf_get_device_number (unsigned short *pIcdev)

Parameter: 	pIcdev: 	[OUT] 	response Device ID

Return:   	return 0 if successful

- 6.1.7 INT WINAPI RF_INIT_TYPE

Function: 	Set Reader contactless working mode

Prototype: 	int WINAPI rf_init_type(unsigned short icdev, unsigned char type)

Parameter: 	icdev:  	[IN]	Device ID

type:   	[IN] 	reader working mode

Return:   	return 0 if successful

Explanation: this function is not effective to the readers only support single protocol.

type = 'A': 	set MIRT-1 into ISO14443A mode

type = 'B': 	set ISO14443B mode

type = 'r': 	set AT88RF020 card mode

type = '1': 	set ISO15693 mode

- 6.1.8 INT WINAPI RF_ANTENNA_STA

Function:  	Manage RF Transmittal

Prototype: 	int WINAPI rf_antenna_sta (unsigned short icdev, unsigned char model)

Parameter: 	icdev: 	[IN] 	Device ID

model:  	[IN] 	transmittal state

Return:   	return 0 if successful

Explanation:model = 0: turn off RF transmittal

model = 1: turn on RF transmittal

- 6.1.9 INT WINAPI RF_LIGHT

Function: 	Manage LED

Prototype: 	int WINAPI rf_light (unsigned short icdev, unsigned char color)

Parameter: 	icdev: 	[IN] 	Device ID

color: 	[IN] 	0 = off

1 = red

2 = green

3 = yellow

Return:   	return 0 if successful

- 6.1.10 INT WINAPI RF_BEEP

Function: 	beep

Prototype: 	int WINAPI rf_beep (unsigned short icdev, unsigned char msec)

Parameter: 	icdev:  	[IN] 	Device ID

msec:  	[IN] 	beep time, unit 10 MSEL

Return:   	return 0 if successful

- 6.2 DES FUNCTION

- 6.2.1 INT WINAPI DES_ENCRYPT

Function: 	DES_Encrypt

Prototype: 	int WINAPI des_encrypt (	unsigned char 	*pSzOut,

unsigned char 	*pSzIn,

unsigned int  	inlen,

unsigned char 	*pKey,

unsigned int  	keylen)

Parameter: 	pSzOut: 	[OUT]	deciphering, bytes length equal to data

pSzIn:   	[IN]	data

inlen:  	[IN]	length of data, integer times of 8 bytes

pKey:   	[IN]	encrypt key

keylen: 	[IN]	length of key, 8 bytes for single DES, 16 bytes for triple DES

Return:   	return 0 if successful

- 6.2.2 INT WINAPI DES_DECRYPT

Function: 	DES_Decrypt

Prototype: 	int WINAPI des_decrypt (	unsigned char 	*pSzOut,

unsigned char 	*pSzIn,

unsigned int  	inlen,

unsigned char 	*pKey,

unsigned int  	keylen)

Parameter:	pSzOut:  	[OUT]	data, bytes length equal to deciphering

pSzIn:    	[IN]	deciphering

inlen:   	[IN]	length of deciphering, integer times of 8 bytes

pKey:     	[IN]	encrypt key

keylen:  	[IN]	length of key, 8 bytes for single DES, 16 bytes for triple DES

Return:   	return 0 if successful

- 6.3 ISO14443A FUNCTION

- 6.3.1 UltraLight

- 6.3.1.1 INT WINAPI RF_REQUEST

Function: 	ReqA

Prototype: 	int WINAPI rf_request (	unsigned short 	icdev,

unsigned char  	model,

unsigned short 	*pTagType)

Parameter: 	icdev:   	[IN] 	Device ID

model:    	[IN] 	REQ MODE

pTagType: 	[OUT]	response data, chip type code

Return:   	return 0 if successful

Explanation:mode = 0x26: REQ_STD

mode = 0x52: REQ_ALL

- 6.3.1.2 INT WINAPI INT RF_UL_SELECT

Function: 	Select UltraLight

Prototype: 	int WINAPI int rf_ul_select (unsigned short icdev,

unsigned char *pSnr,

unsigned char *pLen)

Parameter: 	icdev: 	[IN] 		Device ID

pSnr:   	[OUT]		response data, card unique serial number

pLen:   	[OUT]		length of response data

Return:   	return 0 if successful

- 6.3.1.3 INT WINAPI RF_M1_READ

Function: 	MifareOne read

Prototype: 	int WINAPI rf_M1_read (	unsigned short  icdev,

unsigned char   block,

unsigned char  *pData,

unsigned char  *pLen)

Parameter: 	icdev:  	[IN] 		Device ID

block:  	[IN] 		block absolute address

pData:  	[OUT]		response data from card

pLen:   	[OUT]		length of response data

Return:   	return 0 if successful

Explanation: this function is also applicable for UltraLight card. Every page of UltraLight card has 4 bytes. Transfering this function every time, return data of 4 consecutive pages.

- 6.3.1.4 INT WINAPI INT RF_UL_WRITE

Function: 	UltraLight Write

Prototype: 	int WINAPI int rf_ul_write (	unsigned short  icdev,

unsigned char   page,

unsigned char  *pData)

Parameter: 	icdev: 	[IN]  	Device ID

page:   	[IN]  	UltraLight card page address , 0 ~ 0x0F

pData:  	[IN]	written data, 4 bytes

Return:   	return 0 if successful

- 6.3.1.5 INT WINAPI RF_HALT

Function: 	TYPE_A card HALT

Prototype: 	int WINAPI rf_halt (unsigned short icdev)

Parameter: 	icdev:  	[IN] Device ID

Return:   	return 0 if successful

- 6.3.2 Mifare_Std

- 6.3.2.1 INT WINAPI RF_REQUEST

Function: 	ReqA

Prototype: 	int WINAPI rf_request (	unsigned short  icdev,

unsigned char   model,

unsigned short  *pTagType)

Parameter: 	icdev:    	[IN] 	Device ID

model:    	[IN] 	REQ MODE

pTagType: 	[OUT]	response data, chip type code

Return:   	return 0 if successful

Explanation: mode = 0x26: REQ_STD

mode = 0x52: REQ_ALL

- 6.3.2.2 INT WINAPI RF_ANTICOLL

Function: 	Mifare card Anticollision

Prototype: 	int WINAPI rf_anticoll (	unsigned short  icdev,

unsigned char   bcnt,

unsigned char  *pSnr,

unsigned char  *pLen)

Parameter: 	icdev: 	[IN] 	Device ID

bcnt:  	[IN] 	must be 4

pSnr:  	[OUT]	response data from card, unique serial number

pLen: 	[OUT]	length of response data

Return:   	return 0 if successful

- 6.3.2.3 INT WINAPI RF_SELECT

Function:  	Mifare card Selectting

Prototype: 	int WINAPI rf_select (unsigned short  icdev,

unsigned char  *pSnr,

unsigned char  snrLen,

unsigned char  *pSize)

Parameter: 	icdev: 	[IN] 	Device ID

pSnr: 	[IN] 	card unique serial number

snrLen:   	[IN] 	length of pSnr

pSize:    	[OUT]	response data from card,  capacity code

Return:   	return 0 if successful

Explanation: card will be on active estate after received this command, only one TYPE_A card on active estate at the same influence range at same time.

- 6.3.2.4 INT WINAPI RF_M1_AUTHENTICATION2

Function:  Mifare_Std Authentify

Prototype: int WINAPI rf_M1_authentication2 (	unsigned short  icdev,

unsigned char  model,

unsigned char  block,

unsigned char  *pKey)

Parameter: 	icdev: 	[IN] 	Device ID

model: 	[IN] 	key validate mode

block:   	[IN] 	block absolute address

pKey:   	[IN] 	6 bytes password

Return:   	return 0 if successful

Explanation:model = 0x60: via KeyA

model = 0x61: via KeyB

- 6.3.2.5 INT WINAPI RF_M1_READ

Function: 	MifareOne Read

Prototype: 	int WINAPI rf_M1_read (	unsigned short  icdev,

unsigned char  block,

unsigned char  *pData,

unsigned char  *pLen)

Parameter: 	icdev: 	[IN]	Device ID

block:  	[IN]	block absolute address

pData:  	[OUT]	response data from card

pLen:   	[OUT]	length of response data

Return:   	return 0 if successful

- 6.3.2.6 INT WINAPI RF_M1_WRITE

Function: 	Mifare_Std Write

Prototype: 	int WINAPI rf_M1_write (unsigned short icdev,

unsigned char  block,

unsigned char *pData)

Parameter: 	icdev:   	[IN] 	Device ID

block:  	[IN] 	block absolute address

pData:  	[IN] 	written data, 16 bytes

Return:   	return 0 if successful

- 6.3.2.7 INT WINAPI RF_M1_INITVAL

Function: 	Mifare_Std card Initialize Value

Prototype: 	int WINAPI rf_M1_initval (	unsigned short  icdev,

unsigned char  block,

long  value)

Parameter: 	icdev:   [IN]	Device ID

block:   [IN]	block absolute address

pValue:  [IN] 	initialize purse value at HEX format, low byte in former

Return:   	return 0 if successful

- 6.3.2.8 INT WINAPI RF_M1_READVAL

Function: 	Mifare_Std Read Value

Prototype: 	int WINAPI rf_M1_readval (	unsigned short  icdev,

unsigned char  block,

long *pValue)

Parameter: 	icdev:   [IN] 	Device ID

block:   [IN]	block absolute address

pValue:  	[OUT]	response value at HEX format, low byte in former

Return:   	return 0 if successful

- 6.3.2.9 INT WINAPI RF_M1_INCREMENT

Function: 	Mifare purse increment

Prototype: 	int WINAPI rf_M1_increment (unsigned short  icdev,

unsigned char  block,

long  value)

Parameter: 	icdev:  	[IN] 	Device ID

block:  	[IN] 	block absolute address

value:  	[IN] 	increase value at HEX format, low byte in former

Return:   	return 0 if successful

- 6.3.2.10 INT WINAPI RF_M1_DECREMENT

Function: 	Mifare purse decrement

Prototype: 	int WINAPI rf_M1_decrement (unsigned short  icdev,

unsigned char  block,

long  value)

Parameter: 	icdev:   [IN] 	Device ID

block:   [IN] 	block absolute address

value:   [IN] 	decrease value at HEX format, low byte in former

Return:   	return 0 if successful

- 6.3.2.11 INT WINAPI RF_M1_RESTORE

Function: 	Mifare_Std Restore

Prototype: 	int WINAPI rf_M1_restore (unsigned short icdev, unsigned char block)

Parameter: 	icdev:  	[IN] 	Device ID

block:  	[IN] 	block absolute address

Return:   	return 0 if successful

- 6.3.2.12 INT WINAPI RF_M1_TRANSFER

Function: 	Mifare_Std Transfer

Prototype: 	int WINAPI rf_M1_transfer (unsigned short icdev, unsigned char block)

Parameter: 	icdev:   	[IN] 	Device ID

block:  	[IN] 	block absolute address

Return:   	return 0 if successful

Explanation: this function only be transferred after increment, decrement and restore command

- 6.3.2.13 INT WINAPI RF_HALT

Function: 	Mifare Halt

Prototype: 	int WINAPI rf_halt (unsigned short icdev)

Parameter: 	icdev:  	[IN] 	Device ID

Return:   	return 0 if successful

Explanation: card exit active estate after received this command

- 6.3.3 Mifare_DESFire

- 6.3.3.1 INT WINAPI RF_DESFIRE_RST

Function: 	DESFire Reset

Prototype: 	int WINAPI rf_DESFire_rst (	unsigned short  icdev,

unsigned char  model,

unsigned char  *pData,

unsigned char  *pMsgLg)

Parameter: 	icdev: 	[IN] 	Device ID

model:  	[IN] 	ReqA  mode

pData:   [OUT]	response data from card

pMsgLg: [OUT]	length of response data

Return:   	return 0 if successful

Explanation:mode = 0x26: REQ_STD

mode = 0x52: REQ_ALL

pData = 7 bytes CSN + n bytes RATS according to ISO14443-4 protocol

- 6.3.3.2 INT WINAPI RF_COS_COMMAND

Function: 	DESFire data commutting

Prototype: 	int WINAPI rf_cos_command (	unsigned short  icdev,

unsigned char  *pCommand,

unsigned char  cmdLen,

unsigned char  *pData,

unsigned char  *pMsgLg)

Parameter: 	icdev: 		[IN] 	Device ID

pCommand: 	[IN] 	COS command

cmdLen:    	[IN] 	length of COS command

pData:     	[OUT]	response data from card

pMsgLg:   	[OUT]	length of response data

Return:   	return 0 if successful

- 6.3.4 Mifare_ProX

- 6.3.4.1 INT WINAPI RF_TYPE_RST

Function: 	Request ISO14443A-4 card and reset

Prototype: 	int WINAPI rf_typea_rst (	unsigned short  icdev,

unsigned char  model,

unsigned char  *pData,

unsigned char  *pMsgLg)

Parameter: 	icdev:  	[IN] 	Device ID

model:   	[IN]   	request mode

pData:   	[OUT]	response data from card

pMsgLg: 	[OUT] 	length of response data

Return:   	return 0 if successful

Explanation:mode = 0x26: REQ_STD

mode = 0x52: REQ_ALL

pData: 4bytes CSN + RATS according to ISO14443A

- 6.3.4.2 INT WINAPI RF_ COS_COMMAND

Prototype: 	int WINAPI rf_cos_command (	unsigned short icdev,

unsigned char  *pCommand,

unsigned char  cmdLen,

unsigned char  *pData,

unsigned char  *pMsgLg)

Parameter: 	icdev:  	[IN] 	Device ID

pCommand: 	[IN] 	COS command

cmdLen:    	[IN] 	length of COS command

pData:     	[OUT]	response data from card, including SW1& SW2

pMsgLg:  	[OUT]	length of response data

Return:    return 0 if successful

- 6.3.4.3 INT WINAPI RF_CL_DESELECT

Prototype: 	int WINAPI rf_cl_deselect (unsigned short icdev)

Parameter: 	icdev:      	[IN] 	Device ID

Return:    return 0 if successful

- 6.3.5 SHC1102

- 6.3.5.1 INT WINAPI RF_REQUEST

Function: 	ReqA

Prototype: 	int WINAPI rf_request (	unsigned short 	icdev,

unsigned char  	model,

unsigned short 	*pTagType)

Parameter: 	icdev:   	[IN] 	Device ID

model:    	[IN] 	REQ MODE

pTagType: 	[OUT]	response data from card, chip type code

Return:   	return 0 if successful

Explanation:mode = 0x26: REQ_STD

mode = 0x52: REQ_ALL

- 6.3.5.2 INT WINAPI RF_ SHC1102_AUTH

Function: 	SHC1102 card Authentify

Prototype: 	int WINAPI rf_Shc1102_Auth (unsigned short icdev, unsigned char *pPassword)

Parameter: 	icdev: 	[IN] 	Device ID

pPassword:  	[IN] 	4 bytes password

Return:   	return 0 if successful

- 6.3.5.3 INT WINAPI RF_ SHC1102_READ

Function: 	SHC1102 card read

Prototype: 	int WINAPI rf_Shc1102_Read (unsigned short  icdev,

unsigned char  block,

unsigned char  *pData,

unsigned char  *pLen)

Parameter: 	icdev: 	[IN] 	Device ID

block:  	[IN] 	SHC1102 card block address, 0x00 ~ 0x0F

pData:  	[OUT]	response data from card

pLen: 	[OUT]	length of response data

Return:   	return 0 if successful

- 6.3.5.4 INT WINAPI RF_ SHC1102_WRITE

Function: 	SHC1102 card write

Prototype: 	int WINAPI rf_Shc1102_Write (unsigned short  icdev,

unsigned char  block,

unsigned char  *pData)

Parameter: 	icdev:  	[IN] 	Device ID

block:  	[IN] 	SHC1102 card block address, 0x00 ~ 0x0F

pData:  	[IN] 	written data, 16 bytes

Return:   	return 0 if successful

- 6.4 ISO14443B FUNCTION

- 6.4.1 THR1064

- 6.4.1.1 INT WINAPI RF_TYPEB_RST

Function: 	REQ THR1064 card

Prototype: 	int WINAPI rf_atqb (unsigned short  icdev,

unsigned char  model,

unsigned char  *pData,

unsigned char  *pMsgLg)

Parameter: 	icdev:  	[IN] 	Device ID

model: 	[IN] 	REQ MODE 0=REQB, 1=WUPB

pData:  	[OUT]	response data from card, 8 bytes SN + 4 bytes corresponding data

pMsgLg: 	[OUT]	length of response data

Return:   	return 0 if successful

- 6.4.1.2 INT WINAPI RF_ THR1064_READ

Function: 	THR1064 card read

Prototype: 	int WINAPI rf_thr1064_read(unsigned short  icdev,

unsigned char  page,

unsigned char  *pData,

unsigned char  *pMsgLen)

Parameter: 	icdev: 	[IN] 	Device ID

page:   	[IN] 	page address, 0 ~3

pData:    	[OUT]	response data from card

pMsgLen: 	[OUT]	length of response data

Return:   	return 0 if successful

- 6.4.1.3 INT WINAPI RF_ THR1064_WRITE

Function: 	THR1064 card write

Prototype: 	int WINAPI rf_thr1064_write (unsigned short  icdev,

unsigned char  page,

unsigned char  *pData,

unsigned char  *pMsgLen);

Parameter: 	icdev: 	[IN] 	Device ID

page: 	[IN] 	page address, 0 ~3

pData: 	[IN] 	written data

pMsgLen: 	[OUT]	length of written data

Return:   	return 0 if successful

- 6.4.1.4 INT WINAPI RF_ THR1064_CHECK

Function: 	THR1064 card Authentify

Prototype: 	int WINAPI rf_thr1064_check (unsigned short icdev, unsigned char *pKey)

Parameter: 	icdev: 	[IN] 	Device ID

pKey:  	[IN] 	8 bytes pass word

Return:   	return 0 if successful

- 6.4.2 AT88RF020

- 6.4.2.1 INT WINAPI RF_ TYPEB_RST

Function: 	REQ ISO14443B protocol card and set SLOT

Prototype: 	int WINAPI rf_atqb(unsigned short icdev,

unsigned char  model,

unsigned char  *pData,

unsigned char  *pMsgLg)

Parameter: 	icdev: 	[IN] 	Device ID

model:   	[IN] 	REQ MODE 0 = REQB, 1 = WUPB

pData:   	[OUT]	response data from card

pMsgLg: 	[OUT]	length of response data

Return:   	return 0 if successful

- 6.4.2.2 INT WINAPI RF_ AT020_CHECK

Function:  AT88RF020 card Authentify

Prototype: int WINAPI rf_at020_check (unsigned short icdev, unsigned char *pKey)

Parameter:	icdev: 	[IN] 	Device ID

pKey:  	[IN] 	8 bytes pass word

Return:   	return 0 if successful

- 6.4.2.3 INT WINAPI RF_ AT020_COUNT

Function:  AT88RF020 card count

Prototype: int WINAPI rf_at020_count(unsigned short icdev, unsigned char *pData)

Parameter: icdev:  	[IN] 	Device ID

pData:  	[IN] 	signature, 6 bytes

Return:   	return 0 if successful

- 6.4.2.4 INT WINAPI RF_ AT020_READ

Function: AT88RF020 read

Prototype: int WINAPI rf_at020_read (unsigned short icdev,

unsigned char  page,

unsigned char  *pData,

unsigned char  *pMsgLen)

Parameter: icdev: 	[IN] 	Device ID

page:   	[IN] 	page address, 0 ~ 31

pData:  	[OUT]	response data from card

pMsgLen: 	[OUT]	length of response data

Return:   	return 0 if successful

- 6.4.2.5 INT WINAPI RF_ AT020_WRITE

Function: 	AT88RF020 write

Prototype: 	int WINAPI rf_at020_write (	unsigned short icdev,

unsigned char  page,

unsigned char  *pData)

Parameter: 	icdev: 	[IN] 		Device ID

page:  	[IN] 		page address, 0 ~ 31

pData: 	[IN] 		written data, 8 bytes

Return:   	return 0 if successful

- 6.4.2.6 INT WINAPI RF_ AT020_LOCK

Function: 	AT88RF020 LOCK

Prototype: 	int WINAPI rf_at020_lock (unsigned short icdev, unsigned char *pData)

Parameter: 	icdev: 	[IN] 		Device ID

pData: 	[IN] 		4 bytes data

Return:    return 0 if successful

- 6.4.2.7 INT WINAPI RF_ AT020_DESELECT

Function: 	AT88RF020 card Deselect

Prototype: 	int WINAPI rf_at020_deselect (unsigned short icdev)

Parameter: 	icdev: 	[IN] 		Device ID

Return:    return 0 if successful

- 6.4.3 SR176SRIX4K

- 6.4.3.1 INT WINAPI RF_ST_SELECT

Function: 	ST card (SR176/SRIX4K) Lock

Prototype: 	int WINAPI rf_st_select (unsigned short icdev, unsigned char *pChip_ID)

Parameter: 	icdev:    	[IN] 		Device ID

pChip_ID: 	[IN] 		response data from card, 1 byte ID code

Return:   	return 0 if successful

- 6.4.3.2 INT WINAPI INT_RF_SR176_READBLOCK

Function: 	SR176 Read

Prototype: 	int WINAPI int rf_sr176_readblock (	unsigned short icdev,

unsigned char  block,

unsigned char  *pData,

unsigned char  *pLen)

Parameter: 	icdev:  	[IN] 		Device ID

block:  	[IN] 		block address

pData:  	[OUT]	response data from card

pLen:   	[OUT]	length of response data

Return:   	return 0 if successful

- 6.4.3.3 INT WINAPI INT_RF_SR176_WRITEBLOCK

Function: 	SR176 Write

Prototype: 	int WINAPI int rf_sr176_writeblock (unsigned short icdev,

unsigned char  block,

unsigned char  *pData)

Parameter: 	icdev:  	[IN] 	Device ID

block:  	[IN] 	block address

pData:  	[IN] 	written data, 2 bytes

Return:   	return 0 if successful

- 6.4.3.4 INT WINAPI INT_RF_SR176_PROTECTBLOCK

Function: 	SR176 Lock

Prototype: 	int WINAPI int rf_sr176_protectblock (unsigned short icdev, unsigned char lockreg)

Parameter: 	icdev:   	[IN] 	Device ID

lockreg:	[IN] 	LOCKREG

Return:   	return 0 if successful

Explanation:SR17 6has 16 blocks, every lockreg controls 2 blocks

lockreg      BLOCK                 bit_setting

b7        14 & 15     0:Write Enable    1:Block set as ROM

b6        12 & 13     0:Write Enable    1:Block set as ROM

b5        10 & 11     0:Write Enable    1:Block set as ROM

b4         8 & 9      0:Write Enable    1:Block set as ROM

b3         6 & 7      0:Write Enable    1:Block set as ROM

b2         4 & 5      0:Write Enable    1:Block set as ROM

b1         2 & 3      0:Write Enable    1:Block set as ROM

b0         0 & 1      0:Write Enable    1:Block set as ROM

- 6.4.3.5 INT WINAPI INT_RF_SRIX4K_ GETUID

Function: 	SRIX4K Get UID

Prototype: 	int WINAPI int rf_srix4k_getuid ( unsigned short  icdev,

unsigned char  *pUid,

unsigned char  *pLen)

Parameter: 	icdev:  	[IN] 	Device ID

pUid:    	[OUT]		response data from card, UID

pLen:    	[OUT]		length of response data

Return:   	return 0 if successful

- 6.4.3.6 INT WINAPI INT_RF_SRIX4K_ READBLOCK

Function: 	SRIX4K Read

Prototype: 	int WINAPI int rf_srix4k_readblock (unsigned short icdev,

unsigned char  block,

unsigned char  *pData,

unsigned char  *pLen)

Parameter:	icdev:  	[IN] 	Device ID

block:  	[IN] 	block address

pData:   	[OUT]	response data from card

pLen:   	[OUT]	length of response data

Return:   	return 0 if successful

- 6.4.3.7 INT WINAPI INT_RF_SRIX4K_ WRITEBLOCK

Function: 	SRIX4K Write

Prototype: 	int WINAPI int rf_srix4k_writeblock(unsigned short icdev,

unsigned char  block,

unsigned char  *pData)

Parameter:	icdev:  	[IN] 	Device ID

block:  	[IN] 	block address

pData:  	[IN] 	written data, 4bytes

Return:    return 0 if successful

- 6.4.3.8 INT WINAPI INT_RF_SRIX4K_ PROTECTBLOCK

Function: 	SRIX4K Lock

Prototype: 	int WINAPI int rf_srix4k_protectblock(unsigned short icdev, unsigned char lockreg)

Parameter: 	icdev: 	[IN] 	Device ID

Lockreg: 	[IN] 	LOCKREG

Return:   	return 0 if successful

Explanation: 7~15 blocks of SRIX4K card can be written protect

lockreg      BLOCK                 bit_setting

b7          15        	1:Write Enable    0:Block set as ROM

b6          14        	1:Write Enable    0:Block set as ROM

b5          13        	1:Write Enable    0:Block set as ROM

b4          12        	1:Write Enable    0:Block set as ROM

b3          11        	1:Write Enable    0:Block set as ROM

b2          10        	1:Write Enable    0:Block set as ROM

b1           9        	1:Write Enable    0:Block set as ROM

b0         7 & 8      	1:Write Enable    0:Block set as ROM

- 6.4.3.9 INT WINAPI RF_ST_COMPLETION

Function: 	ST Desactived

Prototype: 	int WINAPI rf_st_completion (unsigned short icdev)

Parameter: 	icdev: 	[IN]		Device ID

Return:   	return 0 if successful

- 6.4.4 TYPE_B SmartCard

- 6.4.4.1 INT WINAPI RF_ TYPEB_RST

Function: 	Req ISO14443B-4 protocol Smart card and Reset

Prototype: 	int WINAPI rf_atqb (	unsigned short  icdev,

unsigned char  model,

unsigned char  *pData,

unsigned char  *pMsgLg)

Parameter: 	icdev: 	[IN] 		Device ID

model: 	[IN] 		REQ MODE 0 = REQB, 1 = WUPB

pData: 	[OUT]	response data from card

pMsgLg: 	[OUT]	length of response data

Return:   	return 0 if successful

- 6.4.4.2 INT WINAPI RF_ COS_COMMAND

Prototype: 	int WINAPI rf_cos_command (unsigned short  icdev,

unsigned char  *pCommand,

unsigned char  cmdLen,

unsigned char  *pData,

unsigned char  *pMsgLg)

Parameter: 	icdev: 	[IN] 		Device ID

pCommand: 	[IN] 		cos command

cmdLen: 	[IN] 		length of cos comman

pData:     	[OUT]	response data from card, including SW1, SW2

pMsgLg:   	[OUT]	length of response data

Return:   	return 0 if successful

- 6.4.4.3 INT WINAPI RF_ CL_DESELECT

Function: 	ISO14443B card Deselect

Prototype: 	int WINAPI rf_cl_deselect (unsigned short icdev)

Parameter: 	icdev:      	[IN] 		Device ID

Return:   	return 0 if successful

- 6.5 ISO15693 FUNCTION

- 6.5.1 INT WINAPI ISO15693_INVENTORY

Function: 	ISO15693_Inventory  (single card)

Prototype: 	int WINAPI ISO15693_Inventory (	unsigned short icdev,

unsigned char *pData,

unsigned char *pLen)

Parameter: 	icdev:   	[IN] 	Device ID

pData:  	[OUT]	response data from tag, 1 byte DSFID + 8 bytes UID

pLen:   	[OUT]	length of response data

Return:   	return 0 if successful

- 6.5.2 INT WINAPI ISO15693_INVENTORYS

Function: 	ISO15693_Inventory  (several cards)

Prototype: 	int WINAPI ISO15693_Inventorys (unsigned short icdev,

unsigned char  *pData,

unsigned char  *pLen)

Parameter: 	icdev:   	[IN] 	Device ID

pData:  	[OUT]   response data from tag, every 9 bytes is a team, the structure of

every team is:

1byte DSFID + 8 bytes UID

pLen: 	[OUT]   	length of response data

Return:   	return 0 if successful

- 6.5.3 INT WINAPI ISO15693_GET_SYSTEM_INFORMATION

Function:  	ISO15693_Get_System_Information

Prototype: 	int WINAPI ISO15693_Get_System_Information(unsigned short icdev,

unsigned char  model,

unsigned char  *pUID,

unsigned char  *pData,

unsigned char  *pLen)

Parameter: 	icdev: 	[IN] 		Device ID

model:   	[IN] 		bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID:  	[IN] 		8 bytes UID

pData:   	[OUT]		response data from tag

pLen:   	[OUT]		length of response data

Return:   	return 0 if successful

Explanation: 	If set Select_flag, only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

Clear Option_flag = 0

- 6.5.4 INT WINAPI ISO15693_SELECT

Function: 	ISO15693_Select

Prototype: 	int WINAPI ISO15693_Select (unsigned short icdev, unsigned char *pUID)

Parameter: 	icdev:  	[IN] 	Device ID

pUID: 	[IN]	8 bytes UID

Return:   	return 0 if successful

- 6.5.5 INT WINAPI ISO15693_RESET_TO_READY

Function: 	ISO15693_Reset_To_Ready

Prototype: 	int WINAPI ISO15693_Reset_To_Ready (unsigned short icdev,

unsigned char model,

unsigned char *pUID)

Parameter:	icdev:  	[IN] 	Device ID

model:  	[IN] 	bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID:   	[IN] 	8 bytes UID

Return:   	return 0 if successful

Explanation: 	If set Select_flag, only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

Clear Option_flag = 0

- 6.5.6 INT WINAPI ISO15693_STAY_QUIET

Function: 	ISO15693_Stay_Quiet

Prototype: 	int WINAPI ISO15693_Stay_Quiet (unsigned short icdev, unsigned char *pUID)

Parameter: 	icdev:  	[IN] 		Device ID

pUID:  	[IN]		8 bytes UID

Return:   	return 0 if successful

- 6.5.7 INT WINAPI ISO15693_GET_BLOCK_SECURITY

Function: 	ISO15693_Get_Block_Security

Prototype: 	int WINAPI ISO15693_Get_Block_Security (	unsigned short  icdev,

unsigned char  model,

unsigned char  *pUID,

unsigned char  block,

unsigned char  number,

unsigned char  *pData,

unsigned char  *pLen)

Parameter: 	icdev: 	[IN] 	Device ID

model: 	[IN] 	bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID: 	[IN] 	8 bytes UID

block: 	[IN] 	block address

number: 	[IN] 	the number of block to be read, < 0x40

pData:  	[OUT]	response data from tag

pLen: 	[OUT]	length of response data

Return:   	return 0 if successful

Explanation: 	If set Select_flag, only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

Clear Option_flag = 0

- 6.5.8 INT WINAPI ISO15693_READ

Function: 	ISO15693_Read

Prototype: 	int WINAPI ISO15693_Read (	unsigned short  icdev,

unsigned char  model,

unsigned char  *pUID,

unsigned char  block,

unsigned char  number,

unsigned char  *pData,

unsigned char  *pLen);

Parameter: 	icdev: 	[IN] 	Device ID

model: 	[IN] 	bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID: 	[IN] 	8 bytes UID

block: 	[IN] 	block address

number: 	[IN] 	the number of block to be read, < 0x40

pData:  	[OUT]	response data from tag

pLen: 	[OUT]	length of response data

Return:   	return 0 if successful

Explanation: 	If set Select_flag, only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

Clear Option_flag = 0

- 6.5.9 INT WINAPI ISO15693_WRITE

Function: 	ISO15693_Write

Prototype: 	int WINAPI ISO15693_Write (	unsigned short  icdev,

unsigned char  model,

unsigned char  *pUID,

unsigned char  block,

unsigned char  *pData)

Parameter:	icdev: 	[IN] 	Device ID

model:  	[IN] 	bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID:   	[IN] 	8 bytes UID

block: 	[IN] 	block address

pData: 	[IN] 	written data, 4 bytes

Return:   	return 0 if successful

Explanation: 	If set Select_flag,	 only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

If write TI card, set Option_flag,

If write I.CODE SLI card, clear Option_flag

- 6.5.10 INT WINAPI ISO15693_LOCK_BLOCK

Function: 	ISO15693_Lock_Block

Prototype: 	int WINAPI ISO15693_Lock_Block (unsigned short  icdev,

unsigned char  model,

unsigned char  *pUID,

unsigned char  block)

Parameter: 	icdev: 	[IN] 	Device ID

model:  	[IN] 	bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID: 	[IN] 	8 bytes UID

block: 	[IN] 	block address

Return:   	return 0 if successful

Explanation: 	If set Select_flag, only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

If write TI card, set Option_flag,

If write I.CODE SLI card, clear Option_flag

- 6.5.11 INT WINAPI ISO15693_WRITE_AFI

Function: 	ISO15693_Write_AFI

Prototype: 	int WINAPI ISO15693_Write_AFI (unsigned short  icdev,

unsigned char  model,

unsigned char  *pUID,

unsigned char  AFI)

Parameter: 	icdev: 	[IN] 	Device ID

model:  	[IN] 	bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID: 	[IN] 	8 bytes UID

AFI:    	[IN] 	AFI to be written

Return:   	return 0 if successful

Explanation: 	If set Select_flag, only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

If write TI card, set Option_flag,

If write I.CODE SLI card, clear Option_flag

- 6.5.12 INT WINAPI ISO15693_LOCK_AFI

Function: 	ISO15693_Lock_AFI

Prototype: 	int WINAPI ISO15693_Lock_AFI (	unsigned short  icdev,

unsigned char model,

unsigned char *pUID)

Parameter:	icdev: 	[IN] 	Device ID

model:  	[IN] 	bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID: 	[IN] 	8 bytes UID

Return:   	return 0 if successful

Explanation: 	If set Select_flag, only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

If write TI card, set Option_flag,

If write I.CODE SLI card, clear Option_flag

- 6.5.13 INT WINAPI ISO15693_WRITE_DSFID

Function: 	ISO15693_Write_DSFID

Prototype: 	int WINAPI ISO15693_Write_DSFID (unsigned short  icdev,

unsigned char  model,

unsigned char  *UID,

unsigned char  DSFID)

Parameter: 	icdev: 	[IN] 	Device ID

model:  	[IN] 	bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID: 	[IN] 	8 bytes UID

DSFID:  [IN] 	DSFID to be written

Return:   	return 0 if successful

Explanation: 	If set Select_flag, only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

If write TI card, set Option_flag,

If write I.CODE SLI card, clear Option_flag

- 6.5.14 INT WINAPI ISO15693_LOCK_DSFID

Function: 	ISO15693_Lock_DSFID

Prototype: 	int WINAPI ISO15693_Lock_DSFID (	unsigned short  icdev,

unsigned char  model,

unsigned char  *pUID)

Parameter:	icdev: 	[IN] 	Device ID

model:  	[IN] 	bit0=Select_flag, bit1=Addres_flag, bit2=Option_flag

pUID: 	[IN] 	8 bytes UID

Return:   	return 0 if successful

Explanation: 	If set Select_flag, only the cards on Selected state respond this command

If set Address_flag, only the cards that the UID are congruous will respond

this command

If write TI card, set Option_flag,

If write I.CODE SLI card, clear Option_flag

- 6.6 Function of Infineon Electric Tag

- 6.6.1 INT WINAPI SRF55VP_READ

Function: 	SRF55XXP Read a PAGE

Prototype: 	int WINAPI Srf55vp_Read (	unsigned short  icdev,

unsigned char  *pUID,

unsigned char  page,

unsigned char  *pData,

unsigned char  *pLen)

Parameter: 	icdev:  	[IN] 	Device ID

pUID:   	[IN] 	8 bytes UID

page:   	[IN]    	address

pData:  	[OUT]	response data from tag

pLen:   	[OUT]	length of response data

Return:   	return 0 if successful

- 6.6.2 INT WINAPI SRF55VP_WRITEBYTE

Function: 	SRF55XXP Write 1BYTE

Prototype: 	int WINAPI Srf55vp_WriteByte (	unsigned short  icdev,

unsigned char  *pUID,

unsigned char  page,

unsigned char  byteaddr,

unsigned char  data)

Parameter: 	icdev: 	[IN] 	Device ID

pUID: 	[IN] 	8 bytes UID

page: 	[IN]	address

byteaddr: [IN] 	write the bytes to excursion address of the PAGE, 0 ~ 8

data: 	[IN] 	written data

Return:   	return 0 if successful

- 6.6.3 INT WINAPI SRF55VP_WRITE

Function: 	SRF55XXP Write a page

Prototype: 	int WINAPI Srf55vp_Write (	unsigned short  icdev,

unsigned char  *pUID,

unsigned char  page,

unsigned char  *pData)

Parameter: 	icdev: 	[IN] 	Device ID

pUID: 	[IN] 	8 bytes UID

page: 	[IN]	address

pData:  	[IN] 	written data, 8 bytes

Return:   	return 0 if successful

- 6.6.4 INT WINAPI SRF55VP_WRITE_REREAD

Function: SRF55XXP write PAGE and Return to the real data of this PAGE

Prototype: int WINAPI Srf55vp_Write_Reread(unsigned short 	icdev,

unsigned char  	*pUID,

unsigned char  	page,

unsigned char  	*pWdata,

unsigned char  	*pRdata,

unsigned char  	*pLen)

Parameter:	icdev: 	[IN] 	Device ID

pUID: 	[IN] 	8 bytes UID

page: 	[IN] 	address

pWdata: 	[IN] 	written data, 8bytes

pWdata: 	[OUT]	response data from tag

pLen: 	[OUT]	length of response data

Return:   	return 0 if successful

- 6.7 PASS THROUGH FUNCTION

- 6.7.1 INT WIN API RF_TRANSCEIVE1

Function: 	Send parameteres to Tag and receive response data

Prototype: 	int WINAPI rf_transceive1(unsigned short 	icdev,

unsigned char  	*pTxData,

unsigned char  	sendLen,

unsigned char  	*pRxData,

unsigned char  	*pMsgLg)

Parameter: 	icdev: 	[IN]  	Communication device identifier

pTxData: 	[IN]  	parameter sent to tag, without CRC bytes

CRC bytes is auto managed by reader

sendLen: 	[IN]  	length of parameter

pRxData: 	[OUT] 	response data from tag

pMsgLg: 	[OUT] 	length of response data

Return: 	return 0 if successful